Irrigation Source Development Optimization

Isabel I Hinton

Department of Civil and Environmental Engineering, Utah State University

CEE 6410: Civil Engineering Systems Analysis in the Age of Uncertainty

October 19, 2026

Introduction: The goal of this analysis is to find the optimal way to develop sources of water for irrigation to maximize profit. A river supplies water for three potential projects. A dam (either high or low) can be built to create a reservoir with a gravity diversion. There is also opportunity for a pump downstream of the reservoir location. This analysis considers the capital, operational costs, and potential earnings from each project over the course of a year (two seasons). In this report, I will describe the model I used and the optimal solution I found for which projects should be built and how much water they should supply each season. I will also give my recommendations and insights from the solution.

Model Formulation: The information present in the problem can be sorted based on two dimensions (or categories): where the water is sourced from (the high dam reservoir, low dam reservoir, or the water pump) and when the water is sourced (season 1 or season 2). The decisions that will affect the profit are which projects are built and how much water is sourced from each project each season. The profit is the sum of the revenue generated minus the costs. The revenue is the total volume of water sourced multiplied by the number of acres irrigated per acft and the revenue per acre. There is varying capital costs associated with building each project. There is also an operational cost per ac-ft of water pumped from the river. There are physical limitations to how the projects can be developed and implemented.

Each project that is built has a maximum capacity. If it has not been built, the project’s capacity is zero. Only one reservoir and therefore size of dam can be built. The maximum volume of water that can be pumped from the river is the inflow into the river, minus the water diverted from the reservoir to farmers, plus flow from the groundwater. The maximum diversion from the reservoir is the inflow for that season.  A mathematical form of the model can be found in Appendix A.
Solution: To produce the optimal amount of profit according to this model formulation, the high dam and water pump should be built. Then, the entire inflow to the reservoir should be diverted and all the ground water pumped for irrigation. The maximum profit that can be extracted using this strategy is $765,400. This solution was found by coding the mathematical model formulation into a GAMS program (Hinton 2026).

Insights and Interpretations: Using the optimal strategy, none of the diversions are at their maximum capacity, so this optimal strategy would still be feasible if the inflow increased. Because groundwater can only be utilized by the pump and is not reliant on the inflow scenario, building the pump is always optimal regardless of the inflow. Because there are no operating costs associated with the reservoir's diversions, once a reservoir is built it is more profitable to divert the entire inflow than pump the water downstream. In this model, I did not constrain the minimum storage in the reservoir or flow in the river below the water pump. Including these realistic constraints would decrease the maximum potential profit and act like a decrease in the seasonal inflow. The above insights would still hold, and the shape of the solution would likely stay the same.

Conclusion: The optimal method of developing this river for irrigation based on the assumed inflows and costs is to build a high dam and downstream pump. Then, divert the entire inflow to the reservoir and pump all the groundwater to the farmers. This strategy balances the initial and operational costs, incorporates the physical limitations of each potential project, and produces the maximum profit.

AI Usage: No AI was used in the analysis or write-up of this report.

References

Hinton (2026). "GAMS Solution for Optimization of Irrigation Development." Github. https://github.com/IzzyH1/CEE-5410---Hinton/blob/main/Mixed%20Integer%20Problem/MixedIntegerHW5.gms

Appendix A: Mathematical Model Formulation

Dimensions: Water sources (source) = {high dam (hd), low dam (ld), river diversion (rd)}, Time (t) = {1, 2}

Decision Variables: What water sources to build and how much water to source from each location in each season

Isource [0 = not built, 1 = build]

Objective: The total profit is the revenue generated per acre irrigated multiplied by the number of the acres irrigated minus the costs to develop the water sources and to operate them.

Max P = source,tRev*dt*Xsource,t - sourceccsource*Isource-source,tcsource*Xsource, t

Rev = The revenue generated per irrigated acre [$] = $300

dt = irrigation demands per acre at each time step [acft/acre]

ccsource = capital cost to develop each source [$]

csource = cost per acft of water from each source [$/acft]

Table 1: Capital, operational cost, and capacity associated with each source

|  | Capital Cost | Operational Cost | Capacity (acft) |
| --- | --- | --- | --- |
| High Dam | $10,000 | 0 | 700 |
| Low Dam | $6,000 | 0 | 300 |
| Water Pump | $8,000 | $20 / acft | 401.5 |

Constraints:

The water extracted from each source must be less than the capacity if the capital cost has been paid. If the capital cost has not been paid, zero water can be extracted from the source

Xsource, t≤capsource* Isource∀source,t

Only one dam can be built

Ihigh dam + Ilow dam≤ 1

Mass balance – The maximum water that can be extracted from the river diversion is the Inflow (Qt) minus the water diverted from the reservoir plus the groundwater (730 acft/yr) Xriver diversion, t≤ Qt - Xhigh dam, t - Xlow dam, t + G ∀t

The maximum volume of water that can be extracted from the reservoir is the Inflow at each time step

Xhigh dam, t +Xlow dam, t ≤Qt∀t

Table 2: Inflow and water demand for each season

|  | Inflow (acft) | Water Demand (acft/acre) |
| --- | --- | --- |
| Season 1 | 600 | 1 |
| Season 2 | 200 | 3 |

| Category (Max. Score) | No Evidence | Doesn’t Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Title (1) | Absent 0 | Evidence of two or less 0 | Evidence of three 0 | Evidence of four 1 | Title – can assess main point from title alone; Name, Instructors’ Names, Course, Date, Neatly finished 1 | 1 |  |
| Introduction (3) | Absent, no evidence 0 | There is no clear introduction or main topic. 1 | Introduction states the main topic but either: - Does not give a full overview, Or: - Too detailed, leading to annoying repetition later. 2 | The introduction states the main topic and previews the structure of the report. 2 | The introduction states the main topic and previews the structure of the report. Good overview of the problem and solution approach. Gives enough detail to motivate the reader to continue reading. 3 | 3 |  |
| Organization and structural development of the idea: procedure, results, conclusions (10) | No content provided. 0 | Paragraphs fail to develop the main idea. No section headers or guide to help the reader understand how material is organized. 1 – 5 | Organization of ideas not fully developed. Paragraphs lack supporting detail sentences. No transitions and/or ineffective section headers. 6 - 7 | Paragraph development present but not perfected. Each paragraph has sufficient supporting detail sentences. Few transitions. 8 | Writer demonstrates logic and sequencing of ideas through well-developed section headers, paragraphs, and transitions. The first sentence of each paragraph is the summary sentence. 9 - 10 | 10 |  |
| Technical Correctness (70) | Questions not addressed. 3 – 42% | The writer has no clue what they are talking about. 45 – 58% | Sketchy: left out required design points. Did not work on this as much as you should have, and it shows. Many important answers are incorrect. 61 – 79% | Discussion lacks adequate detail, but all the necessary points are covered and nearly all answers are correct. 82 – 88% | Provides what was explicitly asked for. The function of each piece is demonstrated to the reader in adequate, but not overwhelming, detail. Answers are correct and reasonable. 91 – 100% |  |  |
|  | - Dimensions and Decision Variables (10) |  |  |  |  | 10 |  |
|  | - Objective Function (10) |  |  |  |  | 10 |  |
|  | - Constraints (10) |  |  |  |  | 10 |  |
|  | - Solution Method (20) |  |  |  |  | 18 |  |
|  | - Results, Recommendations, and New Insights for Management (20) |  |  |  |  | 20 |  |

| Category (Max. Score) | No Evidence | Doesn’t Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Word Usage and Format (7) | Not applicable | Numerous and distracting errors in punctuation, capitalization, spelling, sentence structure, word usage, significant figures, tables, and figures. Data vomited onto page(s). Unacceptable / unprofessional at the graduate level. 1 – 5 | Misspelled words, poor English grammar and word choice. Main body of report is either longer or significantly less than one page. Figures are too small and/or under-labeled, although they are usually of acceptable quality and focus. Tables incoherent or not cohesive. Bad font sizes. Too much or too little data in appendices. Could be improved by being more meticulous. 5 | Almost no errors in punctuation, capitalization, spelling, sentence structure, word usage, significant figures, and presentation of figures, tables, and appendices. Main body of report is one page or less 6 | Punctuation, capitalization, spelling, sentence structure, word usage, and significant figures all correct. Main body of report is one page or less. Clear, consistent fonts. Good word processing skills. Figures have adequate contrast. Informative figure and table titles and legends. Figures have appropriate axis tick spacing, labels, units, and legends. Table columns cohesive, labeled, and specify units. Document is stapled. Appendices, if provided, are separated by topic, and each have a title, discussion, and proper formatting and display of information 7 | 6 |  |
| Conclusion (4) | Absent 0 | Incomplete and/or not focused. 1 | The conclusion does not adequately restate the main results. 2 | The conclusion restates the main results. 3 | The conclusion restates the main results, and is an effective summary. 4 | 4 |  |
| Use of AI (3) | Absent (Return without grade) |  |  | See Policy on Use in Syllabus If used, list the AI tool], provide a short description of use, how use is within the course policy, and link to the transcript of student-AI interactions. If not used, write “No AI was used in the analysis or write-up of this report.” 3 |  | 3 |  |
| References (2) | Absent 0 | Numerous errors, off-the-wall sources used. 0 | Some errors in citing format; more sources should be cited. 1 | Prior work cited with few errors. 2 | All prior work and data sources are cited in the correct format with no errors. 2 | 2 |  |
| TOTAL (100) |  |  |  |  |  | 97 |  |